LAB3 - AD/DA conversion and audio processing

General

The aim of this lab is to implement a DSP application processing audio signals on the Zybo board.

In this specific use-case the final implementation will acquire audio using the audio codec on the board, make analog-to-digital converted samples available for processing on the ARM-based PS, apply Finite Impulse Response filters to the input stream, send the processed samples in output for digiltal-to-analog conversion and play-back.

We will acquire audio from the LINE IN jack plug and send out audio from the HPH OUT jack plug.

The system uses the Analog Devices SSM2603 Audio Codec (see https://www.analog.com/media/en/technical-documentation/data-sheets/SSM2603.pdf ) integrated in the Zybo as an ADC/DAC.

Step 0: download the workspace and analyze it

Download the reference workspace from here. Please refer to the folder related to your Zybo version.

The hardware system in the workspace uses the following peripherals:

- UART (ps7_uart_1)

- timer

- DDR controller (ps7_ddr_0)

- Switches (SWs_4Bits)

- Leds (LEDs_4Bits)

- Buttons (BTNs_4Bits)

Finally, the system integrates the I2S controller for the device ADC/DAC converter. This device is called axi_i2s_adi_1.

You have a visual representation of the block design here.

Step 1: test the loopback implementation

The SDK workspace includes a C file implementing a main() function which shows how the acquisition of samples takes place and the reproduction through the device axi_i2s.

Examine the listing and test the simple loopback IN-OUT.

Pay attention to the initialization of the codec via I2C and to the blocking acquisition via I2S.

Use the global timer available in the Zynq PS to verify the sampling frequency used in the application.

To access the least significant 32 bits of the cycle counter in the timer it is sufficient to include the file and to use Xil_In32(GLOBAL_TMR_BASEADDR + GTIMER_COUNTER_LOWER_OFFSET); to read the content of the register mapped at address GLOBAL_TMR_BASEADDR + GTIMER_COUNTER_LOWER_OFFSET.

Please, be sure that the print of the timer value does not disturb the measurement.

Take into account the operating frequency of the ARM cores (667 MHz) and the fact that the global timer uses a clock with half the frequency compared to the CPU.

Step 2: Audio Filtering

Implement a filter function  that receives a sample in input and performs a FIR type filtering.

The filtering mode requires to execute a convolution over a sliding window of input samples, using a pre-designed kernel of coefficients, tuned to shape the filter for the application need.

The conceptual functionality of the convolution can be expressed as:

y[n]= h0 x[n] + h1 x[n-1] +… + hN x[n-N]

And represented like this

An example of coefficients to be used for the different filtering to be implemented are given at the top of the main.c file. Please create a flexible function that iterates over a parameterizable number of coefficients/taps.

A more aggressive version of the filter is defined by this other 2 filters:

#define coeffLP -0.008747420411798365, -0.01352684070757768, -0.021069157456114974, -0.02821205662046602, -0.03288466862750655, -0.032820056352546804, -0.026015856418133178, -0.011326253746998683, 0.01118086152569252, 0.039926269347420495, 0.07195575020178693, 0.10331516426959793, 0.12972205191226951, 0.14735052987683003, 0.15353880775461448, 0.14735052987683003, 0.12972205191226951, 0.10331516426959793, 0.07195575020178693, 0.039926269347420495, 0.01118086152569252, -0.011326253746998683, -0.026015856418133178, -0.032820056352546804, -0.03288466862750655, -0.02821205662046602, -0.021069157456114974, -0.01352684070757768, -0.008747420411798365

#define N_LP 29

#define coeffHP   0.05946436587252379,   -0.08266255914551396,  -0.032374303236116855,   0.00216595808715192,   0.02865430955587078,   0.045067235048989344,   0.04435253660179216,   0.02016730464237364,   -0.027703198625664668,   -0.0913071374380985, -0.15595705304807897, -0.2038996657538856,   0.7782265468798992,   -0.2038996657538856,   -0.15595705304807897,   -0.0913071374380985,   -0.027703198625664668,   0.02016730464237364,   0.04435253660179216,   0.045067235048989344,   0.02865430955587078,   0.00216595808715192,   -0.032374303236116855,   -0.08266255914551396,   0.05946436587252379

#define N_HP 25

Other filters may be found here.

Please replace this is the main.c to test the developed filter function

As a first step, use the test vectors included in this file.

Later test it on samples coming from ADC, use the switches to choose the train of samples to be sent out - filtered or clean.

You can use any on-line waveform generators  (e.g. https://www.szynalski.com/tone-generator/) to test the system by generating test tones and check the filtering capabilities on the application.

You can create other FIR filter configurations, if you want to try more experiments, any kind of freely available FIR generator, e.g. the tool available at http://t-filter.engineerjs.com/.

Step 3: measure the filtering execution time

Using the global timer, please measure:

- the execution time of the filter function (if necessary to change the parameters);

- the number of cycles available for processing, given the real time constraint imposed by the sampling frequency;

- determine how many filter taps can be performed in real-time without decreasing the sampling frequency

Repeat the measurements after disabling the cache with the built-in primitives:

Xil_ICacheDisable();
    Xil_DCacheDisable();