# LAB4 - Parallel processing

Starting from the workspace used for the audio processing lab, create two application projects:

- one for Processor 0 (name it Master - tip: rename the main .c file to master.c)

- one for Processor 1 (name it worker - tip: rename the main .c file to worker.c)

The aim of the exercise is to use the two cores to add two vectors A and B in a third vector C, sharing the workload.

Modify the main files as follows:

master.c

#include "xparameters.h"

#include "xil_cache.h"

#include "xil_printf.h"

#include "xtime_l.h"

#include "shared.h"

#include "xil_io.h"

u32  t_start, t_end;

int main(void)

{

// Disable caches to avoid coherency issues for this simple exercise.

Xil_ICacheDisable();

Xil_DCacheDisable();

int i;

while (Xil_In32(XPAR_AXI_GPIO_2_BASEADDR)==0);//wait for the start on button

xil_printf("Core 0: Parallel vector add demo (fixed shared base)\r\n");

// Initialize data in shared memory

for (i = 0; i < ARRAY_SIZE; ++i) {

SHARED->A[i] = i;

SHARED->B[i] = 2 * i;

}

// Init flags

SHARED->done0 = 0;

SHARED->done1 = 0;

SHARED->start = 0;

xil_printf("Core 0: Data initialized, signaling core 1 to start...\r\n");

t_start=Xil_In32(GLOBAL_TMR_BASEADDR + GTIMER_COUNTER_LOWER_OFFSET);

// Signal core 1 to start

SHARED->start = 1;

// Core 0 processes first half

for (i = 0; i < ARRAY_SIZE /2 ; ++i) {

SHARED->C[i] = SHARED->A[i] + SHARED->B[i];

}

SHARED->done0 = 1;

// Wait until core 1 finishes

while (SHARED->done1 == 0) {

// busy-wait

}

t_end=Xil_In32(GLOBAL_TMR_BASEADDR + GTIMER_COUNTER_LOWER_OFFSET);

xil_printf("Core 0: Finished first half, waiting for core 1...\r\n");

// Check correctness

int errors = 0;

for (i = 0; i < ARRAY_SIZE; ++i) {

u32 expected = SHARED->A[i] + SHARED->B[i];

if (SHARED->C[i] != expected) {

errors++;

if (errors < 10) {

xil_printf("Mismatch at %d: got %lu, expected %lu\r\n",

i,

(unsigned long)SHARED->C[i],

(unsigned long)expected);

int idx = i;

xil_printf("Debug at %d: A=%lu, B=%lu, C=%lu, expected=%lu\r\n",

idx,

(unsigned long)SHARED->A[idx],

(unsigned long)SHARED->B[idx],

(unsigned long)SHARED->C[idx],

(unsigned long)(SHARED->A[idx] + SHARED->B[idx]));

}

}

}

xil_printf("Core 0: FATTO. Errors = %d, start = %x, end = %x, duration = %x\r\n",

errors, t_start, t_end, t_end - t_start );

while (1) {

// Idle loop

}

return 0;

}

worker.c

#include "xparameters.h"

#include "xil_cache.h"

#include "xil_printf.h"

#include "shared.h"

#include "xil_io.h"

int main(void)

{

int i;

// Disable caches for this exercise (same reason as core 0).

Xil_ICacheDisable();

Xil_DCacheDisable();

xil_printf("Core 1: Waiting for start signal...\r\n");

while (Xil_In32(XPAR_AXI_GPIO_2_BASEADDR)==0);//wait for the start on button

// Wait until core 0 sets start flag

while (SHARED->start == 0) {

// busy-wait

}

//    xil_printf("Core 1: Starting second half computation\r\n");

// Process second half of the array

for (i = ARRAY_SIZE / 2; i < ARRAY_SIZE; ++i) {

SHARED->C[i] = SHARED->A[i] + SHARED->B[i];

}

SHARED->done1 = 1;

//    xil_printf("Core 1: Done.\r\n");

while (1) {

// Idle loop

}

return 0;

}

Create and include in each application project a shared.h file, use this as a template

shared.h

// shared.h

#ifndef SHARED_H

#define SHARED_H

#include "xil_types.h"

#define ARRAY_SIZE 10

/*

* Choose a DDR address that is:

* - Inside your DDR memory range

* - Not used by your linker script / BSS / heap / stack

*

* 0x00100000 is a common safe choice on many Zynq setups,

* but double-check your memory map and adjust if needed.

*/

#define SHARED_BASE 0x2f100000U

typedef struct {

u32 A[ARRAY_SIZE];

u32 B[ARRAY_SIZE];

u32 C[ARRAY_SIZE];

volatile u32 start;  // 0 = wait, 1 = go

volatile u32 done0;  // set by core 0

volatile u32 done1;  // set by core 1

} shared_mem_t;

#define SHARED ((volatile shared_mem_t *)SHARED_BASE)

#endif // SHARED_H

Modify the linker script as shown below

Please notice that the memory mapping of the DDR segments is now as shown below.

Create a "System debugger" run configuration

Please, in the Target Setup tab, take care that the following operations are performed before launching the debugger.

1. Resets entire system. Clears the FPGA fabric (PL).

2. Program FPGA fabric (PL).

3. Runs ps7_init to initialize PS.

4. Runs ps7_post_config. Enables level shifters from PL to PS. (Recommended to use this option only after system reset or board power ON).: reset entire system

In the Application tab assing the correct .elf file to the correct processor, and untick reset processor and stop at program entry.

Measure execution time and experience with changing sizes of the input array.

Change your code to integrate the following implementation:

void vec_add_serial(u32 * restrict c,const u32 * restrict a,const u32 * restrict b,int n)

{

for (int i = 0; i < n; ++i) {

c[i] = a[i] + b[i];

}

}

Compile by changing the Miscellaneuos options to compiler and linker, to the following string

Compiler:

-c -fmessage-length=0 -MT"$@" -mcpu=cortex-a9 -mfpu=neon -mfloat-abi=hard -std=c99 -mvectorize-with-neon-quad -ftree-vectorize -fopt-info-vec-optimized -fopt-info-vec-missed -ffast-math

Linker:

-mcpu=cortex-a9 -mfpu=neon -mfloat-abi=hard -std=c99 -mvectorize-with-neon-quad -ftree-vectorize -Wl,-build-id=none -specs=Xilinx.spec

Check results in the assembly for vector instructions.