LAB5 - DNN on MNIST

General

This lab aims to create a microserver on the Zybo using a DNN to classify handwritten digit images. Every image has to be recognized as a digit from 0 to 9.

Download the lab package here.

Download the template for your C application from here.

Use as reference any workspace used in labs, starting from Lab 2, to interact with the UART.

Attention: to compile the reference functions you need to add the math library to be included/linked in your project.

Right click on your application project in the Project Explorer tab and select C/C++ Build Settings. Customize your compilation script changing the options available on the configuration tabs: you need to add a -lm to your linker invocation.

Thus, expand the ARM v7 Linux gcc linker group and select the Libraries configuration tab. Click on the Add button and add m as new entry to the list of linked libraries.

Analyze the DNN

Use the app available at https://netron.app/ to visualize the topology of your network. Upload the ONNX file available in the example folder in the lab package and check the sequence of layers to be implemented from the visualized plot.

Create your application

Step 0: Try your implementation of the DNN on sample images, by using input and weights pre-loaded as header files in your code. Add the following files to the src folder in your application project.

test_images.h

weights.h

Step 1: Compose your DNN code. The C template file provided above declares few DNN functions that you may use at this aim. Create a sequence corresponding to what visualized in Netron and using adequate buffer arrays for storing inputs/outputs. Depending on how you define this arrays, they may be existing in the stack of the main function, so you may have to change the linker script to increase the size of the stack section.

Step 2: Reusing the content of your implementation in Lab 2, prepare the code for receiving the input for the DNN. You may use RealTerm to send files from your computer to the board.

To compose the short int using two consecutive bytes received from the UART use the following order

data1 = inbyte();  // first received byte

data2 = inbyte();  // second received byte

data_16 = (data2<<8) + data1;

Step 3: Create a while loop implementing the cyclic execution of the classification. The loop iteration will:

receive the image (28x28 pixels)

process it with the composed DNN implementation

print in output the result of the classification.

Step 4: Measure the execution time of the loop iteration and of the single actions in each iteration.

Repeat for your custom network topology

Download the file here

Students are divided in 5 groups. Each students can find his group number on the basis of his/her matriculation number, with groupnumber = matrnumber mod 5.  You will find group-specific files to use in this part of the assignment inside the folder related to your group number. You can disregard folders related to other groups.

Repeat the composition of the network topology and add repeat the testing on the example images.

Tips for optimization

Please check here Tips and tricks for optimization.pdf